Possible Project Ideas to select from:

- Building a compiler for a simple programming language. The programming language is designed to perform simple integer operations such as addition, subtraction, and multiplication. The compiler should translate a high-level mathematical language into MIPS assembly code.

For example:

x = 10 + 5 is translated into:

addi $t0, $zero, 10     # Load constant 10

addi $t1, $zero, 5       # Load constant 5

add $s0, $t0, $t1   # Store result in $s0 (representing variable x)

- Building a MIPS assembler that supports R-type, I-type, and J-type for integer values only. The Assembler converts MIPS assembly language into its equivalent 32-bit binary machine code (or hexadecimal representation). The MIPS Green Sheet and Chapter 2 of this class are great resources to start with.

See https://www.eg.bucknell.edu/~csci320/mips_web

For example:

addi $t0, $s1, 5  is translated into:

001000 10001 01000 000000000000010 (Binary)

Or 0x22280005 (Hexadecimal)

- Building a MIPS Disassembler that reverses machine code into human-readable R-type, I-type, and J-type instructions. The project focuses on "unpacking" 32-bit binary or hexadecimal values to identify opcodes, function codes, and register numbers for integer operations. Students will use bit-masking and shifting techniques to decode the instruction fields and map them back to their assembly mnemonics. The MIPS Green Sheet and the Instruction Encoding tables from Chapter 2 will be essential for identifying the correct instruction formats and register names.

See https://www.eg.bucknell.edu/~csci320/mips_web

For example:

001000 10001 01000 000000000000010 (0x22280005) is translated into:

addi $t0, $s1, 5

- More coming soon…